Biometric Attendance System [Face, Fingerprint] – Greentech Office Complex

1. Introduction

A Biometric Management System at Greentech Office Complex is an advanced IoT-enabled solution that uses facial recognition and fingerprint scanning to automate employee attendance and access control. The system combines multi-modal biometrics to deliver high accuracy, reduce buddy punching, and ensure secure entry management across all floors of the building.

The system operates through an edge+cloud IoT pipeline involving real-time data capture, processing, secure storage, and remote access via a centralized dashboard.

2. Architecture of System & Principles of Operation

Overall Flow of Operations Data Acquisition → Processing → Storage → Access

Working Details

- Data Acquisition

  - Face image captured via high-resolution RGB camera

  - Fingerprint scanned via capacitive sensor at entry terminals

- Processing

  - Facial landmarks and fingerprint minutiae extracted

  - Raw data converted to encrypted biometric templates

- Storage

  - Templates stored in local server and synced to cloud

  - Each employee assigned a unique biometric ID

- Access

  - HR/Admin views live attendance via web dashboard

  - Automated reports generated daily/weekly/monthly

3. Face Recognition: Sensing Devices & Equipment

Face recognition technology employs high-end imaging devices.

Main Components

- Camera (RGB Camera)

  - Captures high-resolution facial images at entry gates

  - Higher resolution increases recognition precision

- IR (Infrared) Sensor

  - Enables face detection in low-light or night conditions

  - Assists in anti-spoofing measures

- Depth Sensor (3D Sensor)

  - Measures face geometry and depth map

  - Guards against photo or video-based spoof attacks

- Processor (Edge Device)

  - Runs real-time face recognition on-site

  - Reduces latency by avoiding full cloud dependency

4. Fingerprint Recognition: Sensors and Modules

Fingerprint recognition uses unique thumb and finger prints for identity verification.

Types of Fingerprint Sensors

- Optical Fingerprint Sensor

  - Captures fingerprint image using light reflection

  - Cost-effective and widely deployed

- Capacitive Fingerprint Sensor

  - Detects fingerprints via electrical signal mapping

  - Higher accuracy and resistance to spoofing

Supporting Modules

- Microprocessor/Controller for real-time matching

- Onboard memory for local fingerprint template storage

- Fingerprint sensors provide high uniqueness and reliability

5. Multi-Modal Biometrics Integration (Face + Fingerprint)

Both modes are integrated to maximize identification efficiency and security.

Integration Process

- Employee presents face and/or fingerprint at terminal

- System processes both biometric inputs simultaneously

- Each mode is matched independently against stored templates

- Final identification decision based on:

  - OR Logic – At least one match grants access (convenience mode)

  - AND Logic – Both must match for entry (high-security mode)

Benefits

- Significantly increased identification accuracy

- Reduced false acceptance and rejection rates

- Stronger protection against spoofing and impersonation

6. Data Flow in the System

Data flow is structured and automated across four key stages:

Step-wise Data Flow Process

- Acquisition – Face image and fingerprint captured at entry terminal

- Feature Extraction – Facial landmarks and fingerprint minutiae extracted by edge processor

- Matching – Extracted features compared against stored biometric templates in database

- Database/Storage – Attendance timestamp, employee ID, and entry logs stored securely

7. Networking, Protocols, and Backend/Cloud Interactions

The system interfaces with local networks and cloud infrastructure for real-time operations.

Network Components

- Wi-Fi and Ethernet connectivity at all entry terminals

- Local server for on-premise backup and cloud server for central management

Protocols Used

- HTTP/HTTPS – Secure communication between terminals and server

- MQTT – Lightweight IoT protocol for real-time event data transfer

- TCP/IP – Core communication protocol for all network activity

Backend/Cloud Tasks

- Secure biometric template storage with AES-256 encryption

- Centralized attendance record management

- Live dashboard and automated reporting

- Remote system monitoring and administration

8. Important Considerations

1. Precision

- Dual-modal recognition achieves over 99% identification accuracy

- Eliminates proxy attendance and buddy punching entirely

2. Latency

- Average recognition time under 2 seconds per employee

- Edge computing ensures fast response without full cloud dependency

3. Anti-Spoofing Features

- IR and 3D depth sensors detect fake or printed face images

- Capacitive fingerprint sensors reject artificial or lifted prints

4. Security and Privacy Measures

- All data transmitted over HTTPS/SSL encrypted channels

- Biometric templates stored in encrypted, access-controlled database

- Role-based access ensures only authorized HR/Admin personnel view records

Conclusion

The Greentech Office Complex Biometric Management System is a robust, IoT-powered solution integrating face recognition and fingerprint scanning for seamless attendance and access control. Through its structured data pipeline and secure cloud connectivity, the system delivers reliable, tamper-proof employee management.

- Uses face + fingerprint (multi-modal biometrics)

- Follows structured data flow (capture → process → store → access)

- Leverages IoT networking with MQTT, HTTPS, and TCP/IP

- Highly accurate, fast, and secure with strong anti-spoofing measures