Lab #3: Understanding Breadboard Usage

Aim

To understand the structure and usage of a breadboard and build simple electronic circuits without soldering.

Theory

A breadboard is a reusable electronic prototyping board used to build and test circuits temporarily without soldering. It helps students and engineers quickly connect components like LEDs, resistors, sensors, and wires.

The breadboard contains interconnected metal strips under the holes. Components inserted into connected rows become electrically connected.

Breadboards are mainly used for:

Circuit prototyping

Learning electronics

Testing circuits before permanent soldering

Breadboard Structure

1. Power Rails

Located on the sides of the breadboard

Marked with + and −

Used for power supply connections

2. Terminal Strips

Middle section of the breadboard

Holes in each row are internally connected

Used for placing components

3. Center Gap

Separates the two halves

Useful for placing ICs (Integrated Circuits)

Components Required

Breadboard

Arduino Uno

LED

220Ω Resistor

Jumper wires

USB cable

Circuit Diagram Explanation

Connections:

Connect Arduino 5V to breadboard + rail

Connect Arduino GND to breadboard − rail

Place LED on breadboard

Connect resistor in series with LED

Connect LED to Arduino digital pin

Procedure

Place the breadboard on the table.

Connect power rails using Arduino 5V and GND.

Insert LED into the breadboard.

Connect resistor to limit current.

Use jumper wires for connections.

Upload LED blink code in Arduino IDE.

Observe LED blinking.

Arduino Code

int ledPin = 13;

void setup() {

pinMode(ledPin, OUTPUT);

}

void loop() {

digitalWrite(ledPin, HIGH);

delay(1000);

digitalWrite(ledPin, LOW);

delay(1000);

}

Observation

LED blinked continuously with a delay of 1 second.

Breadboard connections worked successfully without soldering.

Result

The breadboard was successfully studied and used to build a simple LED circuit without soldering.

Advantages of Breadboard

Reusable

Easy to modify circuits

No soldering required

Useful for beginners and testing

Applications

Electronic circuit prototyping

IoT projects

Arduino experiments

Sensor interfacing

Conclusion

The experiment helped in understanding breadboard connections and building temporary electronic circuits easily and safely .





Lab #8: ESP32 Basics – Blinking Onboard LED

Aim

To study the basics of ESP32 and control the onboard LED by uploading a simple blinking program.

Theory

The ESP32 is a powerful microcontroller developed by Espressif Systems. It is widely used in IoT applications because it provides:

Built-in Wi-Fi

Built-in Bluetooth

Fast processing speed

Low power consumption

ESP32 is more advanced than Arduino Uno and is suitable for smart IoT systems such as:

Smart home automation

Wireless sensor monitoring

IoT dashboards

Cloud-based projects

The onboard LED of ESP32 can be controlled through programming using the Arduino IDE.

Components Required

ESP32 Development Board

USB Cable

Computer/Laptop

Arduino IDE

Software Required

Arduino IDE

ESP32 Board Package

Procedure

Step 1: Install ESP32 Board in Arduino IDE

Open Arduino IDE

Go to File → Preferences

Paste ESP32 board URL in “Additional Board Manager URLs”

Open Boards Manager

Search for ESP32

Install ESP32 package

Step 2: Connect ESP32

Connect ESP32 board using USB cable

Select correct COM port

Choose ESP32 board from Tools → Board

Step 3: Upload Blink Program

Write the LED blink code

Click Upload button

Wait for successful uploading

Observe onboard LED blinking

Arduino IDE Code for ESP32

#define LED_BUILTIN 2

void setup() {

pinMode(LED_BUILTIN, OUTPUT);

}

void loop() {

digitalWrite(LED_BUILTIN, HIGH);

delay(1000);

digitalWrite(LED_BUILTIN, LOW);

delay(1000);

}

Explanation of Code

pinMode() sets LED pin as output

digitalWrite(HIGH) turns LED ON

digitalWrite(LOW) turns LED OFF

delay(1000) creates a 1-second delay

Observation

The onboard LED blinked continuously after uploading the program.

ESP32 successfully executed the code.

Result

The ESP32 board was successfully programmed to blink the onboard LED.

Comparison Between ESP32 and Arduino Uno

Feature	ESP32	Arduino Uno

Processor Speed	240 MHz	16 MHz

Wi-Fi	Available	Not Available

Bluetooth	Available	Not Available

GPIO Pins	More	Fewer

Power Consumption	Low	Moderate

Applications	IoT & Wireless	Basic Electronics

Advantages of ESP32

Built-in Wi-Fi and Bluetooth

High processing speed

Suitable for IoT projects

Supports multiple sensors

Applications

Smart home systems

IoT monitoring

Wireless communication

Automation projects

Conclusion

This experiment introduced the ESP32 microcontroller and demonstrated how to upload code and control the onboard LED. The ESP32 provides advanced wireless communication features, making it highly suitable for modern IoT applications .