PROJECT REPORT ON

TEMPERATURE & HUMIDITY MONITORING SYSTEM WITH VOICE ALERT

Submitted in partial fulfilment of the requirements of the degree of

MASTER OF COMPUTER APPLICATIONS

SUBMITTED BY

| NAME | REG. NUMBER |
| --- | --- |
| Prabin Kumar Nayak | 1ETMCA20251013 |
| Bijayandra Patra | 1ETMCA20251007 |
| Mukesh Jena | 1ETMCA20251017 |
| Satya Ranjan Satpathi | 1ETMCA20251008 |

UNDER THE GUIDANCE OF

MR. BISWAJEEBAN MISHRA

Faculty of Engineering and Technologies

SRI SRI UNIVERSITY
                                               CUTTACK, ODISHA – 754006

# CONTENTS

- Project Overview

- Scope Definition

- Requirements Summary

- System Architecture

- Development Plan

- Testing and Quality Strategy

- Deployment and Operations

- Risk Management

- Change Management

- Conclusion

# Abstract

The Internet of Things (IoT) has changed how we observe and respond to our surrounding environment. This project, titled Temperature and Humidity Monitoring System with Voice Alert, puts these ideas into practice by building a working environmental monitoring setup using a DHT11 sensor and an Arduino UNO microcontroller.

The system reads temperature and humidity values from the DHT11 sensor at regular intervals. These readings are passed to a connected PC through serial communication over a USB cable. On the PC side, a Python script picks up the incoming data, checks it against preset temperature limits, and uses the pyttsx3 text-to-speech library to speak out a relevant alert to the user.

When the temperature goes above 37°C, the system announces a high-temperature warning. If it falls below 20°C, a cold alert is spoken aloud. For temperatures within the acceptable range, a general informational message plays. To avoid unnecessary repetition, the system only triggers a new voice alert when the temperature state actually changes.

This project shows how hardware sensors, serial data transfer, and Python-based processing can be combined to build a practical, inexpensive monitoring tool that requires no manual checks by the user.

# 1. Project Overview

## Background and Motivation

Keeping track of temperature in day-to-day spaces is more important than it might seem. Prolonged exposure to extreme heat or cold can affect people’s health, work output, and general comfort. Traditional ways of monitoring temperature rely on someone physically checking a thermometer or display, which is neither reliable nor practical in all settings. The goal of this project was to replace that manual process with something automatic.

Using a DHT11 sensor paired with an Arduino UNO, the system continuously collects temperature and humidity readings and passes them to a laptop running a Python script. That script listens to the incoming data, evaluates it, and speaks an alert through the PC speakers whenever the temperature crosses a safe threshold – all without any input from the user.

## Objectives

- Design and build a temperature and humidity monitoring system using IoT hardware.

- Read live sensor data from a DHT11 sensor through an Arduino UNO.

- Process the incoming serial data on a PC using a Python script.

- Generate automated voice alerts based on temperature thresholds.

- Keep the overall hardware cost low enough for use in educational settings.

## Success Criteria

- Temperature and humidity are read accurately and without interruption.

- Sensor data is correctly transmitted from the Arduino to the PC over serial.

- Voice alerts are triggered appropriately when thresholds are crossed.

- The system runs continuously without crashing or needing restarts.

- The same alert is not repeated unless the temperature state has changed.

## Stakeholders

Students and researchers working on IoT or embedded systems are the primary audience for this project. Beyond academics, it can also be useful for home users who want a simple way to monitor room temperature, as well as healthcare environments where temperature control plays a critical role. Faculty members and evaluators assessing the project are also key stakeholders from an academic standpoint.

# 2. Scope Definition

## A. What Is Included

- Real-time reading of temperature and humidity from the DHT11 sensor.

- Serial communication between the Arduino UNO and a PC via USB.

- Python-based parsing of the serial data stream and threshold comparison.

- Voice alert generation using the pyttsx3 text-to-speech engine.

- State tracking logic to avoid repeating the same alert multiple times.

## B. What Is Not Included

- Cloud storage or remote monitoring over the internet.

- Mobile app notifications or dashboards.

- Any integration with HVAC or cooling/heating systems.

- AI-based environmental prediction or analysis.

- Visual output via LCD or OLED display modules.

## C. Assumptions and Constraints

The system was built with a few working assumptions in place. A stable USB power source is assumed to be available throughout operation. The Arduino board is expected to be connected on COM6 at a baud rate of 9600. The DHT11 sensor is assumed to give reasonably accurate readings under normal indoor conditions.

On the constraint side, the DHT11 sensor has inherent accuracy limits (±2°C for temperature and ±5% for relative humidity), which means this setup is not suitable for applications requiring precision-grade measurements. Voice alerts also depend on PC speakers being present and functional. Finally, this is a prototype and does not use industrial-grade components.

# 3. Requirements Summary

## A. Functional Requirements

- The DHT11 sensor must take continuous temperature and humidity readings.

- The Arduino UNO must read those values and send them over the serial port.

- The Python script must receive and decode the incoming serial data to extract temperature.

- Temperature must be compared against two thresholds: 37°C (too hot) and 20°C (too cold).

- Voice alerts must only fire when the temperature moves into a new state, not on every reading.

- Any errors during execution must be handled without bringing the entire program down.

## B. Non-Functional Requirements

Reliability is a key concern — the system must run without interruption throughout its operation. In terms of response time, the delay between a sensor reading and the spoken alert should stay under three seconds. The architecture also allows for future expansion: additional sensors such as CO2 or motion detectors could be added without major rewrites. The Python script is written in a modular way to make future updates straightforward. Total hardware cost has been kept under ₹500, making the project accessible for student projects.

## C. Hardware Components

| Component | Qty | Purpose |
| --- | --- | --- |
| Arduino UNO | 1 | Microcontroller board |
| DHT11 Sensor | 1 | Temperature & humidity sensing |
| Jumper Wires | 3 | Circuit connections |
| Arduino USB Cable | 1 | Power and serial communication |
| PC / Laptop | 1 | Runs the Python voice alert script |

# 4. System Architecture

The overall design follows a layered IoT structure, broken into four stages: Sensing, Processing, Communication, and Output. Each stage feeds into the next, forming a clean pipeline from sensor reading to spoken alert.

## 4.1 Data Flow Overview

At the sensing end, the DHT11 sensor picks up ambient temperature and humidity. That data is read by the Arduino UNO and sent as a formatted text string over serial communication at 9600 baud on COM6. A Python script running on the connected PC reads this stream, pulls out the temperature value, and decides which alert to play using the pyttsx3 library. The final output is a spoken message through the PC’s speakers.

## 4.2 Layer-by-Layer Description

The Sensing Layer is handled entirely by the DHT11 digital sensor. It measures temperature in the 0–50°C range and relative humidity between 20–90% RH. Communication with the Arduino happens through a single data wire, with a 10kΩ pull-up resistor keeping the line stable.

In the Processing Layer, the Arduino UNO runs a sketch that initialises the DHT library, takes sensor readings every two seconds, and formats them as strings (for example: Humidity:82.00, Temp:34.20) before printing them over the serial port at 9600 baud.

The Communication Layer is simply the USB cable connecting the Arduino to the PC. Python’s pyserial library reads and decodes the incoming byte stream, making the data available for processing.

The Output Layer is where the Python script does its main work. After parsing the temperature from the serial string, it compares the value against the two thresholds. If the temperature state has changed since the last reading, pyttsx3 generates a platform-native voice alert. This state-tracking approach prevents the same message from being repeated every two seconds.

## 4.3 Working Procedure

- The DHT11 sensor takes continuous readings of temperature and humidity.

- The Arduino reads sensor output every two seconds and formats it as a serial string.

- This string is transmitted to the connected PC over USB serial (COM6, 9600 baud).

- The Python script reads and decodes the incoming stream, extracting the temperature.

- The temperature is checked: 37°C or above triggers a hot alert; 20°C or below triggers a cold alert; anything in between plays a normal status message.

- The alert is only spoken if the temperature state has changed from the previous reading.

- The loop repeats with a one-second sleep between iterations.

# 5. Development Plan

## Methodology

The team followed an Agile approach throughout the project, working in short sprints that each focused on a specific aspect of the build. This made it straightforward to test each component before moving on to the next.

## Sprint 1 – Requirement Analysis

- Reviewed project goals and identified the sensing and communication requirements.

- Selected hardware components: DHT11, Arduino UNO, USB cable, and jumper wires.

- Researched the DHT library and how serial communication works with Python.

## Sprint 2 – Hardware Setup

- Connected the DHT11 sensor to the Arduino UNO using jumper wires.

- Installed and configured Arduino IDE 2.3.8.

- Checked all hardware connections and verified the power supply.

## Sprint 3 – Software Development

- Wrote the Arduino sketch to read DHT11 values and print them over serial.

- Developed the Python script to read and parse the incoming serial data.

- Integrated pyttsx3 to generate voice alerts from text strings.

- Added state-tracking logic so alerts are not unnecessarily repeated.

## Sprint 4 – Testing and Debugging

- Tested sensor accuracy and confirmed data was being transmitted correctly.

- Verified that voice alerts fired correctly at each threshold.

- Fixed serial communication errors and added exception handling.

- Tuned the polling interval and delay values.

## Sprint 5 – Finalisation and Documentation

- Completed hardware and software integration.

- Prepared this project report and supporting documentation.

- Conducted the project demonstration and review.

# 6. Testing and Quality Strategy

## Test Cases

| Test Case | Expected Outcome | Result |
| --- | --- | --- |
| Temperature >= 37°C | Voice: "Warning! Temperature is high" | Pass |
| Temperature <= 20°C | Voice: "Temperature is cold" | Pass |
| Normal range (21–36°C) | Voice: "Temperature is normal" | Pass |
| Serial data reading | Data parsed and printed correctly | Pass |
| Duplicate state | Alert not repeated for same state | Pass |
| Hardware connection | Arduino detected on COM6 at 9600 baud | Pass |

## Acceptance Criteria

The project was considered complete and functional when all of the following conditions were met:

- The DHT11 sensor reliably reads temperature and humidity values.

- The Python script correctly extracts the temperature from the serial string.

- Voice alerts are triggered promptly whenever a threshold is crossed.

- No duplicate messages are played for the same temperature state.

- The system handles runtime errors without crashing.

# 7. Deployment and Operations

The working prototype consists of the Arduino UNO connected to the DHT11 sensor through three jumper wires, with the board powered and communicating via a USB cable plugged into a laptop. The Python script runs on that same laptop inside VS Code.

Once started, the script runs continuously in a loop, checking the temperature every second and delivering a voice alert whenever the state changes. During testing, the terminal output consistently showed readings such as Humidity: 82.00%, Temp: 34.20 Celsius in successive cycles, confirming stable operation.

## Possible Future Additions

- Add an LCD or OLED screen to show temperature and humidity visually without needing a PC.

- Send sensor data to a cloud platform like ThingSpeak or Firebase for logging and remote access.

- Build a mobile app to push temperature alerts to a smartphone.

- Add a buzzer or alarm module directly on the circuit for hardware-level alerts.

- Include humidity-based alerts alongside the existing temperature thresholds.

- Create a web dashboard to view historical sensor data over time.

# 8. Risk Management

| Risk | Possible Impact | How It Was Addressed |
| --- | --- | --- |
| Sensor misreading | Incorrect temperature or humidity values reported | Periodic calibration of the DHT11 sensor |
| Serial port conflict | Python fails to connect to the Arduino | Ensure the COM port is free before running the script |
| Power interruption | System shuts down mid-operation | Use a stable USB power supply from the laptop |
| Python script crash | Voice alerts stop being generated | Wrapped in try/except to log errors and keep running |

# 9. Change Management

before being applied to the running system. The team kept a basic change log recording who made a change, when it Any changes to the hardware wiring, the Arduino sketch, or the Python script should be documented was made, what was changed, and how it tested afterward.

Version control using Git is recommended for tracking code changes going forward. All modified components should be tested in isolation before being deployed to the full integrated setup, to make it easier to catch problems before they affect the whole system.

# 10. Conclusion

This project set out to build a simple, low-cost IoT system capable of monitoring indoor temperature and humidity and alerting users through voice when conditions move outside a comfortable range. That goal was achieved.

The DHT11 sensor attached to the Arduino UNO reliably supplies continuous environmental readings. These are passed over serial to a Python script, which evaluates each reading and uses pyttsx3 to speak a relevant alert: a warning when temperature is too high (37°C or above), a cold alert when it drops too low (20°C or below), and a normal status message otherwise. By tracking the previous state, the system avoids flooding the user with repeated messages.

Developed by four MCA students, the project required hands-on work across embedded C++ for the Arduino, Python scripting, serial communication, and text-to-speech synthesis. The total hardware cost came to under ₹500, keeping it well within reach for student projects.

Looking ahead, the system could be extended in several practical directions — cloud integration, a mobile interface, additional sensors, or a web-based data dashboard. As it stands, the project meets its stated objectives and offers a solid starting point for more advanced environmental monitoring work.