ASSIGNMENT 5: All about pins DIGITAL PINS,ANALOG PINS,PWM-ENABLED PINS

DIGITAL PINS: In arduino didgital pins are like switches they are either in high voltage stage means ON or in low voltage stage means OFF. Basically  it act as input in default , it can act as an output also.

MODE OF OPERTION: Digital pins are versatile in nature as they can be configured to act as both INPUT to read signals from SENSORS and OUTPUT to control devices like LED.

- INPUT:- Digital pins start by listening to voltage . By default they act as an input that’s why they are said to be in HIGH IMPRNDANCE STATE. HIGH IMPENDANCE STATE means it’s like a silent listener that read the voltage without pushing or pulling electricity in the circuit.it tells what the buttons or ports are  doing. It takes very little current to move the input from one state to another.

- OUTPUT:- In output mode digital pin are in LOW IMPENDANCE STATE that means they have very little resistance this allows them to provide enough power to run small gadgets like LED. When the pin set to HIGH the output is aprroax 5v or 3.2v dependding on the board and when set to LOW the pin output is 0v.

- INPUT_PULLUP:-When digital pin is set to INPUT_PULLUP it act as an input withan internal pull-up resistor enabled.It automatically read the HIGH signal by default because of that intrernal resistor.

ANALOG PINS: Analog pins are input pins on a microcontroller that can read continuous voltage signals rather than just two states (HIGH/LOW).Unlike digital pins, which detect only 0 or 1, analog pins measure a range of voltage values.if the voltage varies from 0V to 5V, analog pins can detect any value within this range.

How analog pins handle continuous signals: Analog signals are continuous in nature (they can take infinite values). However, microcontrollers cannot directly process continuous signals. So, the analog pin Receives the continuous voltage and Sends it to an internal system called the Analog-to-Digital Conversion (ADC).

Role of ADC (Analog-to-Digital Conversion):

The ADC converts the continuous analog signal into a discrete digital value that the microcontroller can understand.

Resolution

Resolution defines how many discrete values the ADC can represent.

Example: 10-bit ADC (common in Arduino)

Total values = 2¹⁰ = 1024 levels (0–1023)

Mapping

Mapping means converting the input voltage range into digital values.

For example: 0V → 0 ,5V → 1023 ,2.5V → ~512

So, the ADC maps voltage to numbers between 0 and 1023.

Types of Components/Sensors Connected to Analog Pins

Analog pins are used with sensors that provide varying voltage output. Examples include:

- Temperature sensors (e.g., LM35)

- Light sensors (LDR – Light Dependent Resistor)

- Potentiometers

- Gas sensors

- Pressure sensors

- Humidity sensors

- Sound sensors (microphones)

These sensors produce signals that change continuously based on environmental conditions.

Challenges in Using Analog Pins

a) Noise

- Electrical noise can distort signals.

- Causes inaccurate readings.

- Sources: nearby electronics, power supply fluctuations.

b) Accuracy

- Depends on ADC resolution and sensor quality.

- Limited resolution means small changes may not be detected.

c) Reference Voltage

- ADC uses a reference voltage (e.g., 5V or 3.3V).

- If the reference voltage fluctuates, readings become unstable.

- A stable reference is essential for accurate results.

PWM-Enabled Pins: Concept and Working

What are PWM-Enabled Pins?

PWM-enabled pins are digital output pins on a microcontroller that can simulate analog output using a technique called Pulse Width Modulation (PWM). Even though these pins can only output HIGH (1) or LOW (0), PWM allows them to behave like they are producing a variable analog voltage.

Concept of PWM (Pulse Width Modulation)

PWM is a method of controlling power by switching a digital signal ON and OFF very quickly.Instead of changing voltage directly, PWM changes the width of the ON time in a signal.

Working of PWM

PWM works by generating a rapid square wave:

- The signal switches between HIGH and LOW repeatedly

- The frequency stays constant

- The duty cycle changes

Because switching happens very fast, connected devices average the signal, making it appear like a steady analog voltage.

Example:

- 25% duty cycle ≈ low voltage effect

- 75% duty cycle ≈ high voltage effect

How PWM Enables Analog-Like Control Using Digital Pins

Although digital pins only output 0V or full voltage (e.g., 5V):

- PWM changes how long the signal stays ON

- Devices respond to the average voltage over time

Formula (approx.):
Average Voltage = Duty Cycle × Maximum Voltage

So:

- 50% duty cycle on 5V → ~2.5V effect

- 75% duty cycle → ~3.75V effect

This is how PWM mimics analog output without a true DAC (Digital-to-Analog Converter).

Components That Can Be Connected to PWM Pins

PWM pins are suitable for devices that respond to average power or voltage, such as:

a) LEDs

- Control brightness

- Higher duty cycle → brighter LED

b) DC Motors

- Control speed

- Higher duty cycle → faster rotation

c) Fans

- Speed control (similar to motors)

d) Buzzers/Speakers

- Generate tones by varying frequency and duty cycle

e) Servo Motors

- Control position using specific PWM signals

f) Heaters (via driver circuits)

- Control temperature by adjusting power

Integrated Analysis of Digital, Analog, and PWM Pins

Difference in Signal Type

- Digital pins work with discrete signals. They can only represent two states, HIGH or LOW, which means ON or OFF. There are no values in between.

- Analog pins work with continuous signals. They read voltages that can vary smoothly within a range, such as 0V to 5V. These signals represent real-world changes like temperature or light.

- PWM pins are digital in nature, but they create a signal that behaves like an analog signal using Pulse Width Modulation. They do this by rapidly switching between HIGH and LOW states and controlling how long the signal stays ON.

Difference in Functionality

- Digital pins are mainly used for simple input and output operations. They are used to read switches or control devices that only need ON/OFF action.

- Analog pins are used for input purposes. They take signals from sensors and convert them into digital values using Analog-to-Digital Conversion so that the microcontroller can process them.

- PWM pins are used for output where variable control is required. They help in adjusting things like brightness, speed, or intensity by changing the duty cycle of the signal.

Difference in Applications

- Digital pins are used in applications like buttons, LEDs (simple ON/OFF), and relays.

- Analog pins are used with sensors such as temperature sensors, light sensors, and gas sensors, where continuous data is needed.

- PWM pins are used in applications like controlling LED brightness, motor speed, fan speed, and servo motor position.

Working Together in a Real-World System

- In a practical system, all three types of pins work together.

- First, analog pins are used to sense the environment. For example, a light sensor gives a varying voltage based on light intensity. This voltage is converted into a digital value using ADC.

- Next, the microcontroller processes this data according to the program. It decides what action to take based on the input.

- Finally, output is given through digital or PWM pins. If only simple control is needed, a digital pin is used. If smooth or gradual control is required, a PWM pin is used. This completes the cycle of sensing, processing, and actuation.

Design Trade-offs and Limitations

- Each type of pin has its own advantages and limitations.

- Digital pins are simple and fast but cannot handle continuous values.

- Analog pins can read real-world signals, but they are affected by noise and have limited resolution. Their accuracy also depends on a stable reference voltage.

- PWM pins provide flexible control, but they do not generate true analog signals. In some cases, extra components like filters are needed to smooth the output.

- While designing a system, it is important to choose the correct type of pin depending on the requirement of the task.