- Installing Node-RED

Step 1: Install Node.js first

→ Go to nodejs.org

→ Download LTS version

Step 2: Install Node-RED

Use command prompt

# Install Node-RED globally

npm install -g --unsafe-perm node-red

# Verify installation

Node -v

Step 3: Start Node-RED

npm install -g --unsafe-perm node-red

node-red

Step 4: Open in Browser

Open browser -> go to:

http://localhost:1880

5. Installing Node-RED Dashboard,Arduino node & serial port

The dashboard , arduino node and serial port is a separate package

# Method 1: Via Node-RED Editor (Recommended)

Step 1 → Open Node-RED (localhost:1880)

Step 2 → Click top-right menu (☰)

Step 3 → Select "Manage Palette"

Step 4 → Click "Install" tab

Step 5 → Search "node-red-dashboard" , ”Arduino node” , ”Serial port”

Step 6 → Click Install button

6. Components Required for This Lab

Hardware:  Arduino uno r3 board , DHT 11/22 sensor , jumper wires , usb cable

Software: Arduino IDE, node.js , node red

- Arduino Code

- #include <DHT.h>  // Include the DHT sensor library

#define DHTPIN 3   // Pin where the DHT11 sensor is connected

#define DHTTYPE DHT11  // Define the type of DHT sensor (DHT11)

DHT dht(DHTPIN, DHTTYPE);  // Initialize the DHT sensor with the specified pin and type

void setup()

{

Serial.begin(9600);  // Start the serial communication to send data to the PC

dht.begin();         // Initialize the DHT sensor to start reading data

}

void loop() {

// Read humidity and temperature data from the sensor

float H = dht.readHumidity();      // Read humidity value

float T = dht.readTemperature();   // Read temperature in Celsius

// Check if the sensor readings are valid (not NaN = Not a Number)

if (isnan(H) || isnan(T)) {

Serial.println("Failed to read from DHT sensor!");  // If invalid, print an error message

} else {

// If valid, print the humidity and temperature in a comma-separated format

Serial.println(String(H) + "," + String(T));  // e.g., "45.6,24.3"

}

delay(2000);  // Wait for 2 seconds before taking the next reading

}

Write this code in arduino check whether output is nproperly showing or not . after checking close the serial monitor .

9. Building Node-RED Flow – Step by Step

Step 1: Open Node-RED Editor

Browser → http://localhost:1880

Step 2: Add Serial Node

-> From left panel, drag "serialt in" node to canvas

-> Double-click to configure:

Serial port: same as shown in arduino

Bud rate:9600

Name:Arduino uno

→ Click Done

Step 3: Add Function Node (Data Processing)

-> Drag "function" node to canvas

-> Double-click and enter:

// Split the incoming serial data by comma

var m = msg.payload.split(',');

// Ensure the data is properly split and contains two values

if (m.length === 2) {

// Parse the humidity (first value) and temperature (second value)

var H = { payload: parseFloat(m[0]) };  // Humidity

var T = { payload: parseFloat(m[1]) };  // Temperature

// Return humidity and temperature as separate messages

return [H, T];

} else {

// If the data doesn't contain two values, return null to indicate a problem

return null;

}

Write this code in on message dsection and go to setup present atleftest side of on message and set the output to 2.

-> Name it: "Humidity & Temperature"

-> Click Done

Step 4: Add Dashboard Gauge Node

->  Drag "gauge" node (under dashboard section)

-> Double-click to configure:

Group  : [Add New Group] “dht sensor”→[add new tab] "Sensor Data"

Label  : Temperature

Units  : °C

Range  : Min 0, Max 100

-> Click Done

Add another gauge node ->

-> Double-click:

Group  : [Add New Group] “dht sensor”→[add new tab] "Sensor Data"

Label  : Humidity

Units  : &

Range  : Min 0, Max 100

-> Click Done

Step 6: Add Debug node

Step 7: Connect the Nodes

Serial->debug and function node

Function->gauge

Step 8: Deploy

-> Click red "Deploy" button (top right)

-> All nodes become active

11. Viewing the Dashboard

Step 1 -> After deploying, open browser

Step 2 ->  Go to: http://localhost:1880/ui

Step 3 -> You will see the dashboard with:

-> Temperature Gauge (circular dial)

-> Humidity Gauge (circular dial)