ASSIGNMENT-5

Q: Arduino enables interaction with real-world systems through Digital Pins, Analog Pins, and PWM-enabled Pins. Provide a detailed analytical discussion explaining how these three categories of pins collectively support sensing, decision-making, and control in embedded and loT systems.

Introduction

Arduino helps us connect the digital world (programs) with the real world (sensors, motors, lights).
It does this using three main types of pins:

i)Digital Pins

ii)Analog Pins

iii)PWM Pins

1. Digital Pins:

What are Digital Pins?

Digital pins work with two signals only:

HIGH (1) → ON (5V)

LOW (0) → OFF (0V)

Example: Turning an LED ON or OFF.

Modes of Operation

INPUT → Used to read data (like a button)

OUTPUT → Used to send signals (like turning ON LED)

INPUT_PULLUP → Internal resistor is used to avoid unstable signals

Why important?
These modes decide whether the pin is sending or receiving data.

Types of Components Connected

- Input Devices:

-Push button

-Switch

-Digital sensors

These send signals to Arduino

b) Output Devices:

-LED

-Buzzer

-Relay

These are controlled by Arduino

Practical Considerations

Arduino pins have current limits (~20mA per pin)

Heavy devices (motors, relays) need driver circuits or transistors

2. Analog Pins

What are Analog Pins?

-Analog pins read continuous values, not just 0 or 1.

- Example: Temperature, light, sound

Role of ADC (Analog to Digital Conversion)

Arduino cannot understand analog directly, so it uses ADC.

-Converts voltage (0–5V) into numbers (0–1023)

-This is called 10-bit resolution

Example:
           2.5V → around 512

Value Mapping

You can convert values:

0–1023 → 0–100 (or any range)

Useful for controlling speed, brightness, etc.

Direct connection can damage the board.

Components Connected

-Temperature sensor

-LDR (light sensor)

-Potentiometer

Challenges

Noise → Disturbs signal

Accuracy → Not 100% exact

Reference voltage → Affects readings

So readings may slightly vary.

3.PWM Pins (Pulse Width Modulation)

What is PWM?

-PWM means turning a digital signal ON and OFF very fast.

-It creates an average voltage, acting like analog.

Working Concept

-Signal switches between HIGH and LOW quickly

-The duty cycle (ON time %) controls output

Example:

50% duty → medium brightness

100% → full brightness

Why PWM is useful?

It allows:

-Controlling LED brightness

-Controlling motor speed

Even though pins are digital, output looks like analog.

Components Connected

-LED (brightness control)

-DC motors (speed control)

-Fans

4. Integrated Analysis

Comparison Table:

| Feature | Digital | Analog | PWM |
| --- | --- | --- | --- |
| Signal Type | 0 or 1 | Continuous | Digital pulses |
| Function | ON/OFF | Measure values | Control intensity |
| Example | LED ON/OFF | Temperature sensor | Motor speed |

How They Work Together (Real Example)

Example: Smart Fan System

- Analog Pin → Reads temperature

- Arduino processes data

- PWM Pin → Controls fan speed

- Digital Pin → Turns fan ON/OFF

This shows:

Input → Processing → Output

Trade-offs & Limitations

Digital → Simple but limited

Analog → Accurate but affected by noise

PWM → Not true analog (approximation)

Engineers choose based on:

Accuracy needed

Cost

Power consumption