Lab #0: Git, Git Bash & Preparing Lab Submission

Aim of Experiment :

To study and understand the fundamentals of version control using Git and Git Bash, and to learn how to properly prepare and submit lab work using a structured workflow.

Explanation

This experiment introduces the concept of version control, which helps track changes in files over time. Git allows multiple versions of a project to be maintained efficiently, while Git Bash provides a command-line interface to execute Git commands. Students learn how to initialize repositories, track changes, and manage submissions effectively.

Objective

To understand the basics of Git version control, Git Bash commands, and proper lab submission workflow.

Tools Required

- Git

- Git Bash

- GitHub account

Theory

Git is a distributed version control system used to track changes in code. Git Bash provides a command-line interface to interact with Git.

Procedure

- Install Git and open Git Bash.

- Configure Git:

git config --global user.name "Your Name"

git config --global user.email "your@email.com"

- Initialize repository:

git init

- Add files:

git add .

- Commit changes:

git commit -m "Initial commit"

- Push to GitHub repository.

Result

Successfully learned Git commands and version control workflow.

# Lab #1: Arduino Basics – Blink Inbuilt LED

Aim of Experiment:

To understand the basic working of Arduino and learn how to control digital output by blinking the onboard LED.

Explanation

This experiment demonstrates the simplest Arduino program. It helps in understanding how microcontrollers execute instructions, control pins, and use delays. The onboard LED connected to pin 13 acts as an output device to visualize the working of the code.

Objective

To blink the onboard LED using Arduino.

Components

- Arduino Uno

- USB Cable

Theory

Arduino uses digital pins to control output devices. The onboard LED is connected to pin 13.

Code

void setup() {

pinMode(13, OUTPUT);

}

void loop() {

digitalWrite(13, HIGH);

delay(1000);

digitalWrite(13, LOW);

delay(1000);

}

Procedure

- Connect Arduino to PC.

- Open Arduino IDE.

- Upload code.

Result

LED blinks at 1-second intervals.

Lab #2: Interfacing External LED with Arduino

Aim of Experiment :

To learn how to interface external components with Arduino and control an LED using digital output pins.

Explanation

This experiment extends the previous one by introducing external hardware components like LEDs and resistors. It teaches circuit building on a breadboard and explains the importance of current-limiting resistors to protect components.

Objective

To control an external LED using Arduino.

Components

- Arduino Uno

- LED

- Resistor (220Ω)

- Breadboard

- Jumper wires

Theory

External LEDs require a resistor to limit current and prevent damage.

Code

int ledPin = 7;

void setup() {

pinMode(ledPin, OUTPUT);

}

void loop() {

digitalWrite(ledPin, HIGH);

delay(1000);

digitalWrite(ledPin, LOW);

delay(1000);

}

Procedure

- Connect LED with resistor to pin 7.

- Upload code.

Result

External LED blinks successfully.

# Lab #4: Temperature & Humidity Monitoring using DHT Sensor

Aim of Experiment:

To interface a DHT sensor with Arduino and measure environmental parameters like temperature and humidity.

Explanation

This experiment introduces sensor interfacing. The DHT sensor collects real-world environmental data and sends it to the Arduino. Students learn how to use libraries, read sensor data, and display it using the serial monitor.

Objective

To measure temperature and humidity using DHT sensor.

Components

- Arduino Uno

- DHT11/DHT22 Sensor

Theory

DHT sensor provides digital output of temperature and humidity.

Code

#include <DHT.h>

#define DHTPIN 2

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(9600);

dht.begin();

}

void loop() {

float temp = dht.readTemperature();

float hum = dht.readHumidity();

Serial.print("Temp: ");

Serial.println(temp);

Serial.print("Humidity: ");

Serial.println(hum);

delay(2000);

}

Procedure

- Connect sensor to Arduino.

- Upload code.

- Open Serial Monitor.

Result

Temperature and humidity values displayed.

# Lab #5: Ultrasonic Sensor Interfacing

Aim of Experiment :

To understand distance measurement using ultrasonic waves and interface an ultrasonic sensor with Arduino.

Explanation

This experiment demonstrates how ultrasonic sensors work using sound wave reflection. The Arduino calculates distance based on the time taken for the echo to return. It introduces concepts like pulse generation and time-based calculations.

Objective

To measure distance using ultrasonic sensor.

Components

- Arduino Uno

- HC-SR04 Sensor

Theory

Ultrasonic sensor measures distance using echo time of sound waves.

Code

int trigPin = 9;

int echoPin = 10;

void setup() {

Serial.begin(9600);

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

}

void loop() {

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

long duration = pulseIn(echoPin, HIGH);

int distance = duration * 0.034 / 2;

Serial.println(distance);

delay(500);

}

Procedure

- Connect sensor pins.

- Upload code.

- Monitor distance.

Result

Distance measured in centimeters.

Lab #6: Communication Protocol Case Study - MQTT 
(Mosquitto)
Aim of Experiment:

To study the MQTT protocol and understand how devices communicate using a publish-subscribe model.

Explanation

This experiment focuses on IoT communication. MQTT is a lightweight protocol where devices publish messages to a broker and other devices subscribe to receive them. Mosquitto acts as the broker managing communication between clients.

Objective

To understand MQTT communication protocol.

Theory

MQTT is a lightweight publish-subscribe protocol using a broker (Mosquitto).

Procedure

- Install Mosquitto.

- Start broker.

- Publish message:

mosquitto_pub -h localhost -t test -m "Hello"

- Subscribe:

mosquitto_sub -h localhost -t test

Result   
Successful message transmission using MQTT.




 Lab #7: Web Application using Node-RED for Sensor Data Visualization
Aim of Experiment:

To build a web-based dashboard for visualizing sensor data using Node-RED.

Explanation

This experiment introduces Node-RED, a flow-based programming tool. It allows users to connect sensors and display real-time data on a web dashboard. Students learn how to create flows, process data, and build interactive visualizations.

Objective

To visualize sensor data using Node-RED dashboard.

Tools

Node-RED

DHT Sensor

Theory

Node-RED provides a flow-based programming interface to connect devices and visualize data.

Procedure

Install Node-RED.

Create flow to receive sensor data.

Use dashboard nodes to display data.

Result

Sensor data displayed on web dashboard.

Conclusion

All experiments were successfully performed. The labs provided hands-on experience with Arduino, sensors, communication protocols, and IoT visualization tools