SRI SRI UNIVERSITY

Assignment

Subject: Internet Of Things

Subject Code: TSK301

Name: Sushree Mohanty

Registration Number: 202500541

Program: MCA

Semester:2nd

Faculty: Mr. Biswajeeban Mishra

# Assignment 1: Popular HTTP Libraries and Their Role in HTTP/1.1 (RFC 7230)

1. Overview of HTTP and Libraries

HTTP (Hypertext Transfer Protocol) is the main communication system used on the web. It defines how data is requested and transferred between a client (like a browser or app) and a server.

The rules for HTTP/1.1 are defined in RFC 7230. However, developers do not directly work with these low-level rules. Instead, they use HTTP libraries, which internally handle the protocol and make web communication easier and safer.

These libraries act as ready-made tools that implement HTTP/1.1 behind the scenes.

2. Common HTTP Libraries in Different Programming Languages

Different programming languages provide their own built-in or external libraries for HTTP communication:

- Python
Popular options include Requests, urllib, and urllib3. Among these, Requests is widely used because it simplifies HTTP operations and automatically manages headers, sessions, and responses.

- Java
Java provides HttpClient (built-in in modern versions) and OkHttp. These libraries support both simple and advanced network operations, including asynchronous calls.

- JavaScript
In browsers, the Fetch API is commonly used, while in Node.js, Axios, node-fetch, and Got are popular. Axios is especially known for automatic JSON handling and request customization.

- Ruby
Ruby uses Net::HTTP as a standard library, while http.rb and HTTPClient offer more flexibility and performance improvements.

- PHP
Guzzle is the most widely used library, along with cURL and Symfony HTTP Client. Guzzle is especially known for middleware support and structured request handling.

- C# (.NET)
HttpClient is the primary built-in library, and RestSharp is also commonly used for API-based communication.

- Go
Go uses the net/http package as its standard library. It is simple, fast, and well-suited for concurrent network operations.

- Swift
iOS and macOS applications commonly use URLSession, while Alamofire provides a more developer-friendly interface.

- Rust
Rust provides reqwest for high-level usage and hyper for low-level control. These support asynchronous and safe networking.

- Perl
Perl uses LWP (libwww-perl) and HTTP::Tiny, which are older but still functional for web communication.

## 3. How These Libraries Work with HTTP/1.1 (RFC 7230)

Although developers see simple function calls, these libraries internally follow the rules defined in HTTP/1.1 specification.

They handle several important tasks:

- Request formation
They build proper HTTP requests such as GET, POST, PUT, and DELETE, including headers, URLs, and request bodies.

- Message formatting
They structure messages according to HTTP/1.1 format, which includes a start line, headers, and body.

- Connection handling
They manage TCP connections and support persistent connections (keep-alive) to improve performance.

- Response processing
They interpret server responses, including status codes like 200, 404, and 500, and extract headers and response data.

- Error management
They handle network failures, timeouts, and invalid responses by providing clear error messages.

- Security support
They implement HTTPS using TLS/SSL and also support authentication methods like tokens, API keys, and basic authentication.

In short, these libraries translate the complex HTTP/1.1 protocol rules into simple programming functions.

## 4. Example Library: Python Requests

One of the most widely used HTTP libraries is Requests in Python. It is popular because it makes web communication very simple and readable.

Instead of manually handling sockets or formatting HTTP messages, developers can perform requests using simple functions.

For example, a GET request can be written in just a few lines:

import requests

response = requests.get("https://api.example.com/data")

print(response.status_code)
print(response.json())

## 5. How Requests Helps Developers

The Requests library simplifies HTTP communication in several ways:

- It hides low-level socket handling

- It automatically formats HTTP requests correctly

- It converts JSON responses into Python objects

- It manages cookies and sessions automatically

- It provides clear error handling for failed requests

Because of these features, developers can focus on application logic instead of networking details.

## 6. Conclusion

- HTTP libraries play a very important role in modern software development. They act as an interface between programmers and the HTTP/1.1 protocol defined in RFC 7230.

- Instead of manually implementing complex networking rules, developers use libraries like Requests, Axios, HttpClient, and Guzzle to communicate with web servers easily and efficiently.

- These tools ensure that web communication is fast, structured, and reliable across different programming languages.

# Assignment 2: Biometric Face Recognition Attendance System (Shruti)

## Ans-1. System Identification

The biometric face recognition attendance system used in the college is an IoT-based automated attendance system installed at entry points or classrooms.

Since detailed vendor information is not provided, it is considered a standard institutional face recognition attendance system used for student identification and attendance marking.

System Name: College Face Recognition Attendance System
Manufacturer/Vendor: Not specified
Installation Location: 
System Type: IoT-based biometric attendance system

## 2. System Architecture

The system works in multiple layers that handle capturing, processing, and storing attendance data.

- Input Layer
A camera captures the face of the student.

- Processing Layer
The system processes the image, detects the face, and extracts features for identification.

- Storage Layer
Attendance data and user records are stored in a local database or cloud server.

- Network Layer
The system uses internet connectivity (Wi-Fi or LAN) to transfer data.

- Application Layer
A dashboard or software interface is used to view attendance records.

Block Flow (for diagram)
User → Camera → Face Detection → Feature Processing → Matching → Database → Attendance Marking

## 3. Sensor Analysis

The system mainly uses a camera for face capture.

Camera Sensor
Type: Image capture sensor
Working: Captures facial image of the user
Role: Primary input for face recognition
Some systems may also use proximity sensing to detect presence before capture.

## 4. Face Recognition Workflow

The system follows these steps:

- Face Detection
The camera identifies whether a face is present in the frame.

- Face Alignment
The face image is adjusted for proper orientation.

- Feature Extraction
Facial features are converted into digital patterns.

- Matching
The system compares extracted features with stored records.

- Attendance Marking
If a match is found, attendance is recorded automatically.

5. Data Flow Analysis

When a student stands in front of the system, the camera captures the face image. This image is processed and compared with stored data in the database. If a match is found, attendance is marked and saved in the system.

The process is mostly real-time, meaning attendance is recorded immediately.

## 6. IoT Integration

The system is connected through a network (Wi-Fi or LAN).

It allows real-time attendance updates and remote access through a dashboard. Data can also be stored in cloud systems or college servers for centralized management.

## 7. Security and Privacy

The system uses biometric data, which is unique to each individual, making it more secure than traditional methods.

Data is stored in a secure database and may be protected using encryption techniques.

However, there are some concerns:

- Privacy of biometric data

- Risk of unauthorized access if security is weak

- Possible errors in recognition under poor conditions

## 8. Performance Evaluation

The system is generally fast and provides quick attendance marking.

Accuracy is high under proper lighting and clear face visibility. However, performance may reduce if lighting is poor or the face is partially covered.

## 9. Limitations and Improvements

Limitations:

- Depends on lighting conditions

- May fail if face is not clearly visible

- Requires stable network connection

- Privacy concerns related to biometric data

Improvements:

- Better low-light performance

- Stronger security for stored data

- Backup system for network failure

- Anti-spoofing (liveness detection)

## 10. Comparative Analysis (Optional)

Face Recognition vs Fingerprint System
Face recognition is contactless and faster, while fingerprint requires physical touch.

Face Recognition vs RFID System
RFID requires cards, which can be lost or shared. Face recognition is more automatic but depends on visibility conditions.

## Conclusion

The biometric face recognition attendance system is an efficient IoT-based solution for automating attendance in educational institutions. It reduces manual work, improves accuracy, and provides real-time tracking. Despite some limitations, it is a useful and modern system for smart campuses.

ASSIGNMENT 3:Smartphone Sensor Profiling and Analysis MOTOROLA EDGE 60 PRO

1. Device Identification

Model: Motorola Edge 60 Pro
Manufacturer: Motorola
Year of Release: 2024–2025 (approx.)
Category: Mid-range / Premium smartphone

2. Operating System Details

OS Name: Android
OS Version: Android 16
UI Layer: Motorola MyUX (near stock Android experience)
Kernel: Linux-based Android kernel

3. Sensor Identification (Using Tools or Apps)

- Accelerometer

- Gyroscope

- Magnetometer (Compass)

- Proximity sensor

- Ambient light sensor

- Fingerprint sensor

- GPS (Location sensor system)

- Microphone (audio input sensor)

- Camera sensors (front and rear)

Screenshot: (Insert from Sensor Box / Device Info HW app)

4. Sensor Classification

Motion Sensors:
Accelerometer
Gyroscope

Position Sensors:
Magnetometer
GPS

Environmental Sensors:
Ambient light sensor
Proximity sensor

Biometric Sensors:
Fingerprint sensor

Imaging Sensors:
Front camera sensor
Rear camera system

5. Sensor Details

Sensor Name: Accelerometer
Type: Motion sensor
Vendor: Varies (commonly Bosch or STMicroelectronics)
Unit of Measurement: m/s²
Range: 78.453606 m/s²
Sampling Rate: High (real-time motion tracking)
Use Case: Screen rotation, step counting, motion detection

Sensor Name: Gyroscope
Type: Motion sensor
Vendor: Bosch / STMicroelectronics
Unit: degrees/second
Range: 34.9063 rad/sec
Sampling Rate: High
Use Case: Gaming, AR, camera stabilization

Sensor Name: Ambient Light Sensor
Type: Environmental sensor
Unit: lux
Range: 65535.0 lx
Use Case: Auto brightness control

Sensor Name: Proximity Sensor
Type: Environmental sensor
Range: short distance detection (0–5 cm approx.)
Use Case: Turning off screen during calls

Sensor Name: Fingerprint Sensor
Type: Biometric sensor
Use Case: Device unlocking and security authentication

6. Sensor Data Observation

Sensor 1 Name: Accelerometer
Reading: X = +2.15 m/s², Y = 3.32 m/s², Z = 8.03 m/s²
Condition: Indoor (phone kept in hand)

Sensor 2 Name: Light Sensor
Reading: +27.00 lx (approx. indoor lighting)
Condition: Indoor room lighting

7. Sensor Fusion

Sensor fusion is the combination of data from multiple sensors to improve accuracy and performance.

For example, the accelerometer and gyroscope work together to detect precise motion and orientation of the phone. GPS and magnetometer combine to improve navigation accuracy in maps. This helps in applications like gaming, fitness tracking, AR apps, and navigation systems.

8. IoT Perspective

Smartphone as Edge Device:
The smartphone collects real-time data from sensors, processes it locally, and responds immediately without needing external systems.

Smartphone as Gateway:
It transfers sensor data to cloud services using Wi-Fi or mobile data and connects with other IoT devices like smartwatches, smart home devices, and Bluetooth sensors.

9. Critical Analysis

Missing Sensors:
leaser autofocus,selfie camera autofocus ,barometer

Power Consumption Issues:
Sensors like GPS and gyroscope consume more battery when used continuously.

Cost vs Sensor Quality:
Higher-end smartphones have more accurate and faster sensors compared to mid-range devices.

Limitations / Accuracy:
Sensor readings may be affected by noise, calibration errors, and environmental interference.

10. Conclusion

The Motorola Edge 60 Pro contains multiple advanced sensors that allow it to interact with the physical environment. These sensors help in motion tracking, navigation, security, and environmental awareness. Overall, the smartphone acts as a powerful IoT edge device by sensing, processing, and transmitting real-world data efficiently.

## ASSIGNMENT 4:Understanding Arduino Uno R3 Pinout (With Explanation)

Ans- The Arduino Uno R3 consists of different types of pins, and each type has a specific role in connecting the board to the external world. These pins allow the Arduino to take input, process it, and give output.

Digital Pins (0–13)

- The Arduino Uno R3 has 14 digital pins. These pins work with only two values, HIGH and LOW.

- When used as input, a digital pin reads signals like whether a button is pressed or not. For example, if a button is pressed, the pin reads HIGH; otherwise, it reads LOW.

- When used as output, the pin sends signals to control components. For example, it can turn an LED ON or OFF.This makes digital pins useful for simple control tasks.

- Pins 0 and 1 have an additional function. They are used for serial communication, which helps Arduino communicate with a computer or other devices.

PWM Pins (3, 5, 6, 9, 10, 11)

- Some digital pins have PWM capability. These pins use Pulse Width Modulation to give an output that behaves like analog.

- Instead of providing a constant voltage, these pins switch ON and OFF very fast. By controlling how long the signal stays ON (duty cycle), they control the average voltage.

- Because of this, PWM pins are used when gradual control is needed. For example,   they can control LED brightness or motor speed smoothly instead of just ON or OFF.

Analog Pins (A0–A5)

- The Arduino Uno R3 has 6 analog pins. These pins are used to read signals that change continuously.

- Sensors like temperature sensors or light sensors give output in the form of varying voltage. The Arduino cannot directly understand this, so it uses Analog-to-Digital Conversion to convert the voltage into a digital number.

- This allows the Arduino to measure real-world values and use them in decision-making.

Power Pins

- Power pins are used to supply voltage to the Arduino and connected components.

- The 5V and 3.3V pins provide power to sensors and modules. The GND pins complete the circuit by providing a common ground. The Vin pin is used when supplying power from an external source like a battery.

- Without proper use of power pins, the circuit will not function correctly.

Communication Pins

- Some pins are used for communication with other devices.

- Pins 0 and 1 are used for serial communication, which allows data exchange with a computer.

- Pins A4 and A5 are used for I2C communication, and pins 10 to 13 are used for SPI communication. These communication methods are important for connecting devices like displays, memory modules, or advanced sensors.

Reset Pin

The reset pin is used to restart the Arduino. When it is activated, the program begins execution from the start. This is useful during testing or if the system needs to be restarted.

AREF Pin

The AREF pin is used to provide a reference voltage for analog inputs. This helps improve the accuracy of analog readings, especially when working with sensors that require a specific voltage range.

## Conclusion

Each pin in the Arduino Uno R3 has a clear purpose. Digital pins handle simple input and output, analog pins read real-world signals, and PWM pins provide smooth control. Power and communication pins support the overall functioning of the system. By understanding how each pin works, it becomes easier to design and implement practical electronic projects.

ASSIGNMENT 5: All about pins DIGITAL PINS,ANALOG PINS,PWM-ENABLED PINS

Ans- DIGITAL PINS: In arduino didgital pins are like switches they are either in high voltage stage means ON or in low voltage stage means OFF. Basically  it act as input in default , it can act as an output also.

MODE OF OPERTION: Digital pins are versatile in nature as they can be configured to act as both INPUT to read signals from SENSORS and OUTPUT to control devices like LED.

- INPUT:- Digital pins start by listening to voltage . By default they act as an input that’s why they are said to be in HIGH IMPRNDANCE STATE. HIGH IMPENDANCE STATE means it’s like a silent listener that read the voltage without pushing or pulling electricity in the circuit.it tells what the buttons or ports are  doing. It takes very little current to move the input from one state to another.

- OUTPUT:- In output mode digital pin are in LOW IMPENDANCE STATE that means they have very little resistance this allows them to provide enough power to run small gadgets like LED. When the pin set to HIGH the output is aprroax 5v or 3.2v dependding on the board and when set to LOW the pin output is 0v.

- INPUT_PULLUP:-When digital pin is set to INPUT_PULLUP it act as an input withan internal pull-up resistor enabled.It automatically read the HIGH signal by default because of that intrernal resistor.

ANALOG PINS: Analog pins are input pins on a microcontroller that can read continuous voltage signals rather than just two states (HIGH/LOW).Unlike digital pins, which detect only 0 or 1, analog pins measure a range of voltage values.if the voltage varies from 0V to 5V, analog pins can detect any value within this range.

How analog pins handle continuous signals: Analog signals are continuous in nature (they can take infinite values). However, microcontrollers cannot directly process continuous signals. So, the analog pin Receives the continuous voltage and Sends it to an internal system called the Analog-to-Digital Conversion (ADC).

Role of ADC (Analog-to-Digital Conversion):

The ADC converts the continuous analog signal into a discrete digital value that the microcontroller can understand.

Resolution

Resolution defines how many discrete values the ADC can represent.

Example: 10-bit ADC (common in Arduino)

Total values = 2¹⁰ = 1024 levels (0–1023)

Mapping

Mapping means converting the input voltage range into digital values.

For example: 0V → 0 ,5V → 1023 ,2.5V → ~512

So, the ADC maps voltage to numbers between 0 and 1023.

Types of Components/Sensors Connected to Analog Pins

Analog pins are used with sensors that provide varying voltage output. Examples include:

- Temperature sensors (e.g., LM35)

- Light sensors (LDR – Light Dependent Resistor)

- Potentiometers

- Gas sensors

- Pressure sensors

- Humidity sensors

- Sound sensors (microphones)

These sensors produce signals that change continuously based on environmental conditions.

Challenges in Using Analog Pins

a) Noise

- Electrical noise can distort signals.

- Causes inaccurate readings.

- Sources: nearby electronics, power supply fluctuations.

b) Accuracy

- Depends on ADC resolution and sensor quality.

- Limited resolution means small changes may not be detected.

c) Reference Voltage

- ADC uses a reference voltage (e.g., 5V or 3.3V).

- If the reference voltage fluctuates, readings become unstable.

- A stable reference is essential for accurate results.

PWM-Enabled Pins: Concept and Working

What are PWM-Enabled Pins?

PWM-enabled pins are digital output pins on a microcontroller that can simulate analog output using a technique called Pulse Width Modulation (PWM). Even though these pins can only output HIGH (1) or LOW (0), PWM allows them to behave like they are producing a variable analog voltage.

Concept of PWM (Pulse Width Modulation)

PWM is a method of controlling power by switching a digital signal ON and OFF very quickly.Instead of changing voltage directly, PWM changes the width of the ON time in a signal.

Working of PWM

PWM works by generating a rapid square wave:

- The signal switches between HIGH and LOW repeatedly

- The frequency stays constant

- The duty cycle changes

Because switching happens very fast, connected devices average the signal, making it appear like a steady analog voltage.

Example:

- 25% duty cycle ≈ low voltage effect

- 75% duty cycle ≈ high voltage effect

How PWM Enables Analog-Like Control Using Digital Pins

Although digital pins only output 0V or full voltage (e.g., 5V):

- PWM changes how long the signal stays ON

- Devices respond to the average voltage over time

Formula (approx.):
Average Voltage = Duty Cycle × Maximum Voltage

So:

- 50% duty cycle on 5V → ~2.5V effect

- 75% duty cycle → ~3.75V effect

This is how PWM mimics analog output without a true DAC (Digital-to-Analog Converter).

Components That Can Be Connected to PWM Pins

PWM pins are suitable for devices that respond to average power or voltage, such as:

a) LEDs

- Control brightness

- Higher duty cycle → brighter LED

b) DC Motors

- Control speed

- Higher duty cycle → faster rotation

c) Fans

- Speed control (similar to motors)

d) Buzzers/Speakers

- Generate tones by varying frequency and duty cycle

e) Servo Motors

- Control position using specific PWM signals

f) Heaters (via driver circuits)

- Control temperature by adjusting power

Integrated Analysis of Digital, Analog, and PWM Pins

Difference in Signal Type

- Digital pins work with discrete signals. They can only represent two states, HIGH or LOW, which means ON or OFF. There are no values in between.

- Analog pins work with continuous signals. They read voltages that can vary smoothly within a range, such as 0V to 5V. These signals represent real-world changes like temperature or light.

- PWM pins are digital in nature, but they create a signal that behaves like an analog signal using Pulse Width Modulation. They do this by rapidly switching between HIGH and LOW states and controlling how long the signal stays ON.

Difference in Functionality

- Digital pins are mainly used for simple input and output operations. They are used to read switches or control devices that only need ON/OFF action.

- Analog pins are used for input purposes. They take signals from sensors and convert them into digital values using Analog-to-Digital Conversion so that the microcontroller can process them.

- PWM pins are used for output where variable control is required. They help in adjusting things like brightness, speed, or intensity by changing the duty cycle of the signal.

Difference in Applications

- Digital pins are used in applications like buttons, LEDs (simple ON/OFF), and relays.

- Analog pins are used with sensors such as temperature sensors, light sensors, and gas sensors, where continuous data is needed.

- PWM pins are used in applications like controlling LED brightness, motor speed, fan speed, and servo motor position.

Working Together in a Real-World System

- In a practical system, all three types of pins work together.

- First, analog pins are used to sense the environment. For example, a light sensor gives a varying voltage based on light intensity. This voltage is converted into a digital value using ADC.

- Next, the microcontroller processes this data according to the program. It decides what action to take based on the input.

- Finally, output is given through digital or PWM pins. If only simple control is needed, a digital pin is used. If smooth or gradual control is required, a PWM pin is used. This completes the cycle of sensing, processing, and actuation.

Design Trade-offs and Limitations

- Each type of pin has its own advantages and limitations.

- Digital pins are simple and fast but cannot handle continuous values.

- Analog pins can read real-world signals, but they are affected by noise and have limited resolution. Their accuracy also depends on a stable reference voltage.

- PWM pins provide flexible control, but they do not generate true analog signals. In some cases, extra components like filters are needed to smooth the output.

- While designing a system, it is important to choose the correct type of pin depending on the requirement of the task.

ASSIGNMENT 6:Embedded Systems Communication Mechanisms –Comparative Study

Ans- Serial vs Parallel Communication

Data Transmission

- Serial: Data is transmitted one bit at a time through a single communication line, which reduces wiring complexity.

- Parallel: Multiple bits are transmitted simultaneously using multiple lines, increasing data transfer per clock cycle.

Speed

- Serial: Slower in terms of bits per cycle, but efficient in real systems due to high clock rates and reduced interference.

- Parallel: Faster for short distances because multiple bits are sent at the same time.

Wiring and Hardware Complexity

- Serial: Requires very few wires, making circuit design simple and cost-effective.

- Parallel: Requires many wires (one for each bit), making the system bulky and complex.

Distance and Reliability

- Serial: More reliable over long distances because timing mismatch is minimal.

- Parallel: Less reliable over long distances due to signal skew and timing differences between wires.

Practical Usage

- Serial: Widely used in embedded systems (UART, I2C, SPI) due to scalability and simplicity.

- Parallel: Used mainly in internal systems like memory buses or processor communication.

Analog vs Digital Communication

- Type of Signal

- Analog: Uses continuous signals that vary smoothly over time (voltage or current changes continuously).

- Digital: Uses discrete signals represented as binary values (0 and 1).

Noise Sensitivity

- Analog: Highly affected by noise and signal distortion, which reduces accuracy.

- Digital: Less affected by noise because signals can be regenerated and corrected.

Accuracy and Stability

- Analog: Less stable and accuracy decreases with distance and interference.

- Digital: More stable and highly accurate due to clear signal levels.

Processing in Microcontrollers

- Analog: Difficult to process directly; usually requires ADC conversion.

- Digital: Easy to process since microcontrollers work on binary data.

Usage in Embedded Systems

- Analog: Mainly used for raw sensor outputs before conversion.

- Digital: Used for almost all communication between devices and modules.

Synchronous vs Asynchronous Communication

- Clock Requirement

- Synchronous: Uses a shared clock signal between sender and receiver.

- Asynchronous: Does not use a shared clock; timing is managed using start and stop bits.

- Data Transfer Method

- Synchronous: Continuous stream of data synchronized with clock pulses.

- Asynchronous: Data is sent in frames with start and stop bits.

- Speed and Efficiency

- Synchronous: Faster and more efficient because there is no extra overhead for each byte.

- Asynchronous: Slightly slower due to additional start and stop bits.

- Complexity

- Synchronous: More complex due to clock synchronization requirement.

- Asynchronous: Simpler to implement and more flexible.

- Examples

- Synchronous: SPI communication.

- Asynchronous: UART communication.

Overall Role in Embedded Systems:

In practical embedded systems, these communication methods are selected based on system requirements. Serial communication is preferred over parallel communication due to lower hardware complexity and better scalability. Digital communication dominates modern systems because of its high reliability and resistance to noise. Synchronous communication is used when high-speed data transfer is required, while asynchronous communication is preferred for simpler and more flexible connections.

Together, these communication techniques allow microcontrollers to effectively interface with sensors, memory devices, communication modules, and external hardware, forming the backbone of real-world embedded system applications.