INDUSTRIAL IoT (IIoT)

AUTOMOTIVE SYSTEM ANALYSIS

Case Study: Connected Electric Vehicle — Chetak 3501

Submitted for: Industrial IoT Systems & Architecture

Academic Year 2025-27

# 1. Introduction

The Bajaj Chetak 3501 is a next-generation connected electric two-wheeler that exemplifies the convergence of Electric Vehicle (EV) technology with Industrial Internet of Things (IIoT) principles. Launched in the Indian market, the Chetak 3501 integrates onboard telematics, a cloud backend, a mobile application, and over-the-air (OTA) update capabilities — placing it firmly in the IIoT automotive domain.

This case study provides a thorough architectural, communicative, and performance analysis of the Chetak 3501's connected ecosystem, addressing all aspects of its IIoT implementation from device sensors to cloud analytics.

| Vehicle | Bajaj Chetak 3501 Connected Electric Scooter |
| --- | --- |

| Category | IIoT-Enabled Electric Two-Wheeler (Consumer IoT / Automotive IIoT) |
| --- | --- |

| Connectivity | 4G LTE / Bluetooth 5.0 / GPS |
| --- | --- |

| Cloud Platform | Bajaj Auto proprietary cloud with AWS/Azure backend |
| --- | --- |

| App Support | Bajaj Chetak Mobile App (Android & iOS) |
| --- | --- |

# 2. Architectural Analysis

The Chetak 3501's IIoT architecture follows the classic four-layer model: Device, Connectivity, Edge, and Cloud/Application. Each layer plays a distinct and critical role in the vehicle's connected intelligence.

## 2.1 Device Layer

The device layer forms the physical foundation of the IIoT system. It consists of all onboard hardware capable of sensing, actuating, and generating data:

- Battery Management System (BMS) — monitors cell voltage, temperature, State of Charge (SoC), and State of Health (SoH).

- Vehicle Control Unit (VCU) — the central onboard computer coordinating motor, braking, and power delivery.

- Inertial Measurement Unit (IMU) — detects tilt, vibration, and sudden deceleration (accident detection).

- GPS/GNSS Module — tracks real-time location, speed, and route history.

- Onboard Diagnostics (OBD-equivalent) — reads fault codes, motor temperature, and controller health.

- Proximity & Anti-theft Sensors — detects unauthorized movement or tampering.

## 2.2 Connectivity Layer

This layer manages how sensor data is transmitted from the vehicle to higher layers. The Chetak 3501 supports multi-protocol connectivity:

- 4G LTE (via embedded SIM) — primary channel for cloud telemetry, OTA updates, and app commands.

- Bluetooth 5.0 — short-range connection to rider's smartphone for proximity unlock, local diagnostics.

- GPS-over-LTE — location data streamed to cloud via LTE backhaul.

## 2.3 Edge Layer

The edge layer in the Chetak 3501 is implemented within the VCU and BMS microcontrollers. Edge intelligence is critical for safety-critical real-time decisions:

- Local fault detection and emergency response (e.g., BMS cuts power if cell overheat is detected without waiting for cloud).

- Local data filtering — the VCU aggregates and compresses raw sensor data before transmission.

- Throttle-by-wire control loop — operates at <10ms latency, entirely local.

- Offline mode — stores ride data locally when network is unavailable and syncs later.

## 2.4 Cloud & Application Layer

The cloud layer provides fleet management, analytics, and user-facing services:

- Telemetry ingestion server — receives MQTT/HTTPS data streams from vehicles.

- Data analytics engine — processes ride statistics, battery health trends, predictive maintenance.

- Remote command execution — processes user commands from mobile app (e.g., lock/unlock, find vehicle).

- Notification microservice — dispatches push notifications, SMS alerts for critical events.

- Mobile App (iOS/Android) — user interface for all vehicle interactions.

- OTA management server — distributes firmware updates to vehicle fleet.

# 3. Edge vs. Cloud Intelligence

## 3.1 Division of Responsibilities

| Criterion | Edge (VCU/BMS) vs. Cloud |
| --- | --- |
| Latency | Edge: <10ms (critical control). Cloud: 100ms–2s (analytics/notifications). |
| Safety-Critical Tasks | Always edge — cannot tolerate network latency. |
| Storage | Edge: Temporary ring buffer (minutes). Cloud: Persistent (years). |
| Compute Power | Edge: Limited MCU (ARM Cortex-M). Cloud: Scalable VMs. |
| Examples at Edge | BMS fault cut-off, traction control, ABS-equivalent, proximity lock. |
| Examples at Cloud | Trip history, battery health trends, remote lock, OTA updates. |

## 3.2 Justification for Safety-Critical Edge Processing

Safety-critical processes MUST be handled at the edge for the following reasons:

- Network unreliability: A cellular connection may drop in tunnels, remote areas, or during handovers. Delegating safety decisions to the cloud creates a single point of failure that could be catastrophic.

- Latency: The BMS must respond to a thermal runaway event in under 10ms. Round-trip cloud latency is typically 150–500ms — orders of magnitude too slow.

- Regulatory compliance: Automotive safety standards (ISO 26262 ASIL-B equivalent) require deterministic, self-contained safety systems.

- Independence: Even if a malicious actor compromises the cloud server, the vehicle's core safety systems must continue to operate correctly.

# 4. Data Flow Analysis

## 4.1 Telemetry Upload (Vehicle → Cloud)

The primary data pipeline flows from the vehicle's sensors to the cloud analytics platform:

- Sensors generate raw data (voltage, temperature, speed, GPS) at 10–100Hz.

- VCU aggregates and filters data, reducing it to 1Hz telemetry payloads.

- Data is serialized (JSON or Protobuf) and published to an MQTT broker over LTE.

- MQTT broker (e.g., AWS IoT Core / Azure IoT Hub) ingests and fans out to subscribers.

- Stream processor (e.g., AWS Kinesis) routes data to time-series database and analytics engine.

- Mobile app queries REST API to display real-time and historical data to the user.

## 4.2 User Commands (App → Vehicle)

Command flow from the mobile application to the vehicle:

- User initiates action (e.g., Remote Lock) in the Chetak mobile app.

- App sends HTTPS POST request to the Bajaj cloud API server.

- API server authenticates token, validates command, publishes to vehicle's MQTT command topic.

- Vehicle's MQTT client receives command, VCU executes (e.g., activates immobilizer relay).

- VCU publishes acknowledgment back to cloud; app receives confirmation push notification.

## 4.3 Event Notifications (Vehicle → User)

Alert-driven notifications for critical events:

- Onboard sensor or VCU detects event (e.g., theft, low battery, motor fault).

- VCU publishes event message to MQTT event topic with priority flag.

- Cloud notification microservice processes event and triggers push notification via FCM/APNs.

- User receives real-time alert on mobile app with event details and recommended action.

# 5. Protocol Stack

The Chetak 3501 employs a layered protocol stack. The following table maps each networking layer to the protocols used:

| Layer | Protocol | Function |
| --- | --- | --- |
| Application | MQTT v3.1.1 / HTTPS (REST) | Telemetry publish, command subscribe, API calls |
| Session/Transport | TLS 1.3 / TCP / UDP | Encrypted sessions; TCP for reliability, UDP for low-latency |
| Network | IPv4 / IPv6 | Internet routing over cellular network |
| Data Link / Physical | 4G LTE (eMBB) / Bluetooth 5.0 / BLE | WAN cellular + local BLE for phone pairing |
| Embedded Protocols | CAN Bus / UART / I2C | Internal vehicle communications between ECUs/sensors |

Protocol Selection Rationale

- MQTT chosen over HTTP for telemetry due to its publish-subscribe model, low bandwidth overhead, and support for QoS levels — ideal for unreliable LTE connections.

- HTTPS/REST used for command and control as it provides reliable, stateless, and easily authenticated request-response semantics.

- TLS 1.3 provides mutual authentication and encryption, protecting against man-in-the-middle attacks over cellular networks.

- CAN Bus is the automotive industry standard for ECU communication — deterministic, fault-tolerant, and well-proven.

# 6. Event-Driven Behavior

The Chetak 3501 implements an event-driven architecture where specific sensor conditions trigger automated actions locally and remotely.

## 6.1 Low Battery Alert

- Trigger: BMS SoC drops below 20% threshold.

- Edge Action: VCU reduces motor peak power output to extend range.

- Cloud Action: Event published to MQTT broker → notification microservice → push alert to user's smartphone with estimated remaining range.

- User Action: App displays nearest charging stations on map.

## 6.2 Motor/Battery Fault Detection

- Trigger: BMS detects cell over-temperature (>60°C) or VCU detects motor controller fault code.

- Edge Action (Immediate): BMS isolates affected cell group; VCU enters limp-home mode.

- Cloud Action: Fault code, timestamp, GPS location, and sensor snapshot uploaded to cloud for analysis.

- User Notification: Alert with fault description, service center recommendation, and whether safe to continue riding.

## 6.3 Theft Detection

- Trigger: IMU detects significant tilt or movement while vehicle is parked/locked.

- Edge Action: Activates immobilizer (electronic lock on motor controller).

- Cloud Action: GPS tracking initiated at higher frequency (1Hz instead of 0.1Hz).

- User Notification: Immediate push alert with live location tracking on app map.

# 7. Performance & Scalability Analysis

## 7.1 Latency Analysis

| Operation | Typical Latency / Performance |
| --- | --- |
| BMS safety cut-off (edge) | <5ms — deterministic, no network dependency |
| Throttle control loop | <10ms — hard real-time, VCU local |
| Telemetry MQTT publish | 50–200ms over LTE — acceptable for monitoring |
| Remote lock command | 200ms–1s — user command, slight delay acceptable |
| OTA firmware download | 5–30 minutes depending on update size and signal |
| Push notification delivery | 1–5 seconds end-to-end |

## 7.2 Bandwidth Consumption

- Telemetry stream (1Hz, JSON): ~200–500 bytes/payload = ~15–35 KB/min = ~20–50 MB/day per vehicle.

- GPS data: ~50 bytes/second at 1Hz = ~4 MB/day.

- OTA update (typical firmware): 5–20 MB per update, compressed and delta-patched to minimize usage.

- Total estimated data per vehicle per day: 50–100 MB in active operation.

## 7.3 Large-Scale Deployment Challenges

- Connection management: At 100,000+ active vehicles, the MQTT broker must handle 100K+ persistent TCP connections. Requires clustering (e.g., EMQ X, HiveMQ cluster) or managed services (AWS IoT Core).

- Ingestion rate: 100K vehicles at 1Hz = 100,000 messages/second. Requires horizontally scalable stream processing (Apache Kafka, AWS Kinesis).

- Database load: Time-series data grows rapidly. Specialized time-series databases (InfluxDB, TimescaleDB) or columnar stores (AWS Timestream) are required.

- OTA rollout: Simultaneous OTA updates to large fleets can overwhelm bandwidth. Staged rollout (canary → 10% → 50% → 100%) is essential.

- Geographical distribution: Vehicles across India need edge PoP (Points of Presence) to reduce cloud latency. CDN-based edge nodes or AWS CloudFront help.

# 8. Security Analysis

## 8.1 Authentication

- Device Authentication: Each vehicle has a unique X.509 digital certificate provisioned at manufacturing. Used for mutual TLS (mTLS) authentication with the MQTT broker.

- User Authentication: Mobile app uses OAuth 2.0 + JWT tokens with short expiry. Multi-factor authentication (OTP via SMS) for account registration.

- API Security: All REST APIs require Bearer token; server-side validation on every request.

## 8.2 Encryption

- Transport Encryption: All vehicle-to-cloud and app-to-cloud communication uses TLS 1.3 — providing forward secrecy, authenticated encryption.

- Data at Rest: Telemetry data stored in cloud databases is encrypted using AES-256.

- OTA Integrity: Firmware updates are signed with manufacturer's private key; vehicle verifies signature before installation using public key stored in secure enclave.

- BLE Communication: Bluetooth pairing uses BLE Secure Connections (LESC) with ECDH key exchange.

## 8.3 Known Vulnerabilities & Mitigations

| Vulnerability | Mitigation Strategy |
| --- | --- |
| MQTT broker DDoS | Rate limiting per client ID; broker clustering with load balancer |
| Replay attacks on commands | Timestamped JWT tokens with 30-second expiry window |
| Physical BLE sniffing | BLE LESC pairing; rolling keys; distance-limited pairing mode |
| Cloud account takeover | OAuth 2.0 + MFA; anomalous login detection; device fingerprinting |
| OTA supply chain attack | Code-signed firmware; secure boot in VCU; version rollback protection |
| GPS spoofing | Cross-validate GPS with accelerometer/gyro data; anomaly detection in cloud |

# 9. Critical Evaluation & MQTT-Based Distributed Alternative

## 9.1 Current Architecture Limitations

- Centralized cloud creates a single point of failure — if cloud goes down, remote features are unavailable.

- High bandwidth costs for large fleets transmitting continuous telemetry.

- Vendor lock-in with proprietary Bajaj cloud platform limits third-party integrations.

- Latency for remote commands depends entirely on cloud availability and LTE signal quality.

## 9.2 Proposed MQTT-Based Distributed Architecture

A distributed MQTT broker federation with regional edge brokers would significantly improve resilience and latency:

- Deploy regional MQTT brokers (e.g., Mumbai, Delhi, Pune, Bangalore) as edge PoPs.

- Vehicles connect to the nearest broker — reduces round-trip latency from 200ms to <50ms.

- Brokers federate (bridge) to a central cloud broker — regional data is synchronized.

- Mobile apps can connect to regional brokers for lower-latency command delivery.

- If central cloud fails, regional brokers continue serving local commands and storing telemetry until reconnection.

## 9.3 Trade-off Analysis

| Dimension | Distributed vs. Centralized Trade-offs |
| --- | --- |
| Availability | Distributed wins — no single point of failure; regional failover. |
| Latency | Distributed wins — regional brokers reduce RTT by 60–80%. |
| Consistency | Centralized wins — distributed introduces eventual consistency challenges. |
| Cost | Distributed costs more — multiple broker deployments, network inter-connects. |
| Complexity | Centralized is simpler — one broker, one management plane. |
| Compliance | Distributed may help data residency — regional data stays in region. |

# 10. Failure Scenarios & System Resilience

## 10.1 Network Failure (LTE Connectivity Loss)

- Behavior: VCU detects no MQTT heartbeat ACK within timeout. System switches to offline mode.

- Impact: Remote commands (lock/unlock, find vehicle) unavailable. Telemetry stored locally in ring buffer.

- Recovery: On network restoration, buffered telemetry uploaded in chronological order using MQTT QoS 1 (at-least-once delivery).

- Safety: All safety-critical functions (BMS protection, motor control) continue unaffected — edge processing is independent of connectivity.

## 10.2 Cloud Server Failure

- Behavior: MQTT broker or API server becomes unreachable.

- Impact: No new notifications, remote commands, or OTA updates. App shows 'Vehicle Offline'.

- Recovery: Cloud employs redundancy — multi-AZ deployment, load balancers, auto-scaling groups. Mean Time to Recovery (MTTR) target: <5 minutes.

- Mitigation: Circuit breaker pattern in vehicle client — retries with exponential backoff (1s, 2s, 4s, 8s...) to prevent thundering herd on cloud recovery.

## 10.3 Device (VCU) Failure

- Behavior: VCU crash or power failure.

- Impact: Complete loss of connected features, motor control, and ride capability.

- Recovery: VCU implements watchdog timer — if firmware hangs >500ms, hardware reset is triggered. Safe-state mode activated (throttle disabled, brakes released).

- Detection: Persistent VCU failures are logged to non-volatile flash. On next power cycle, vehicle uploads diagnostic report to cloud before resuming normal operation.

# 11. Conclusion

## 11.1 Why the Chetak 3501 Qualifies as an IIoT System

The Bajaj Chetak 3501 fulfills all defining criteria of an Industrial IoT system:

- Physical Integration: Sensors and actuators are embedded in an industrial asset (vehicle) operating in real-world physical conditions.

- Real-Time Monitoring: Continuous telemetry of critical parameters (battery, motor, location) with millisecond edge response.

- Remote Control & Automation: Cloud-based remote commands and automated event-driven responses without human intervention.

- Scalability: Architecture supports thousands to hundreds of thousands of connected vehicles.

- Data-Driven Intelligence: Collected data enables predictive maintenance, performance optimization, and user personalization.

- Interoperability: Standard protocols (MQTT, HTTPS, TLS, CAN) ensure integration with third-party services.

## 11.2 Strengths

- Robust safety architecture with edge-first approach for critical decisions.

- Multi-protocol connectivity (LTE + BLE) providing redundancy and flexibility.

- Strong security posture with X.509 certificates, TLS 1.3, and signed OTA updates.

- Scalable cloud backend capable of handling fleet-level analytics and management.

- Offline resilience — vehicle stores data locally and syncs when connectivity returns.

## 11.3 Limitations

- Centralized cloud dependency for all remote features — regional outages affect all users.

- Proprietary platform reduces interoperability with third-party charging/fleet management solutions.

- LTE-only WAN connectivity — no WiFi fallback when in parking locations.

- Limited edge compute — current VCU cannot run ML models for advanced predictive analytics locally.

- No V2X (Vehicle-to-Everything) capability — future iterations should incorporate C-V2X for intersection safety and traffic coordination.

— End of Case Study —